# Design Requirements

Project Name: Navigators
SIH Problem Statement ID: SIH26168

## 1. System Architecture

The system employs a 100% offline edge architecture focused entirely on edge inference to ensure real-time responsiveness and security when GNSS is denied. No backend or cloud dependency is allowed in the runtime.

### 1.1 Edge Engine (Smartphone / External IMU)

- Sensor Interface: Access raw data from smartphone sensors (Accelerometer, Gyroscope) at 10 Hz. Supports up to 200 Hz for external FOG-IMU.

- Preprocessing Module: Employs non-linear median filtering and Butterworth low-pass filtering to isolate and remove physical shocks (e.g., potholes) without distorting kinematic data. Calculates phone-to-vehicle alignment.

- AI Inference Engine: A lightweight neural network (Temporal Convolutional Network or LSTM) trained with rigorous data augmentations (noise injection, dynamic scaling). Designed to predict 2D velocity from windowed IMU data.

- Sensor Fusion (EKF): A 15-state Extended Kalman Filter implemented entirely in JavaScript to optimally fuse AI-predicted velocities with GNSS readings (when available) directly on the edge.

- Kinematic Constraints Module: Enforces Non-Holonomic Constraints (NHC) and Zero Velocity Updates (ZUPT) dynamically.

- Map Matching Module: Snaps estimated diverging trajectories back to known road centerlines using Geometric and Probabilistic (HMM) offline mapping.

- UI/Visualizer: Renders the estimated trajectory continuously without jumping or freezing inside a Progressive Web App (PWA).

### 1.2 Processing Flow

- GNSS Available: GNSS + IMU → AI/ML filtering → Fusion (EKF) → Map matching → Position

- GNSS Lost: IMU → AI speed/motion estimation → Dead reckoning → NHC & ZUPT → Map matching → Position

- GNSS Returns: Dead reckoning → Re-acquisition → GNSS + INS fusion → Corrected continuous position

## 2. Software Requirements

- AI/ML Framework: PyTorch for training; ONNX Runtime Web for 100% offline edge deployment inside the browser.

- Data Processing: Pandas, NumPy, SciPy for preprocessing and filtering.

- Navigation Engine: Pure JavaScript implementation for EKF, NHC, ZUPT, and Map Matching to eliminate Python backend dependencies during deployment.

## 3. Hardware Requirements

- Development: Workstation or Cloud instance with GPU (NVIDIA) for training on the large IO-VNBD dataset.

- Deployment: Modern smartphone (Android/iOS) with capable IMU, utilizing the CPU/NPU for WebAssembly inference, or dedicated edge hardware for external IMU integration.