# Navigators IDR - Implementation Plans & Progress Report

Project: Navigators (SIH26168 for ISRO)
Objective: AI/ML-powered navigation system demonstrating seamless vehicle tracking during GNSS denial using Intelligent Dead Reckoning.

## Phase 1: Core Navigation Engine & AI Model (Completed)

### Plan

- Data Pipeline: Build loaders for the VNBD dataset (IMU & GNSS data) to prepare for model training.

- AI Model: Implement a Temporal Convolutional Network (TCN) in PyTorch to estimate velocity from IMU (accelerometer & gyroscope) data.

- Navigation Engine (EKF): Develop an Extended Kalman Filter (15-state) to fuse GNSS and IMU data.

- Dead Reckoning (DR) Engine: Implement a system that takes over when GNSS is denied, integrating the AI-estimated velocity to update position.

- Constraints: Implement Non-Holonomic Constraints (NHC) and Zero Velocity Update (ZUPT) as pseudo-measurements to limit drift.

### What We Did

- Created src/data/io_vnbd_loader.py and src/data/dataset.py.

- Developed src/models/tcn_model.py and training scripts (scripts/train.py, scripts/evaluate.py).

- Built the navigation logic in src/navigation/ (ekf.py, dead_reckoning.py, nhc.py, zupt.py).

- Wrote extensive unit tests (51/51 passing) verifying components like Haversine distance, EKF updates, and ZUPT logic.

## Phase 2: Web Simulator & Model Export (Completed)

### Plan

- Simulation UI: Create a visual dashboard to demonstrate the system working.

- Visualization: Use Leaflet.js to plot Ground Truth vs. Estimated Trajectory on a map.

- Telemetry: Add gauges for Speed, Heading, Position Error, and Drift Percentage.

- Deployment Prep: Create an export script to convert the PyTorch model to ONNX format for edge deployment.

### What We Did

- Built a web-based simulator (simulator/index.html, index.css, app.js).

- Integrated playback controls to simulate historical data runs.

- Created src/edge/onnx_export.py and successfully exported the PyTorch model.

- Demonstrated automatic mode switching (GNSS -> Dead Reckoning) with visual indicators in the simulator.

## Phase 3: Live Mobile Sensor Integration (Completed)

### Plan

- Backend Server: Build a FastAPI server (src/api/server.py) to receive live data via WebSockets.

- Mobile Frontend: Upgrade the simulator to a Progressive Web App (PWA) with a Service Worker (simulator/manifest.json, simulator/sw.js).

- Sensor Capture: Write live_sensor.js to capture live DeviceMotionEvent (Accelerometer, Gyroscope) and Geolocation (GNSS) from a mobile phone and stream it to the backend.

### What We Did

- Successfully wired up the mobile sensors to the Python backend.

- Deployed a WebSocket pipeline for real-time EKF processing on the Mac, with the mobile device acting as a thin client/sensor node.

## Phase 4: 100% Offline Edge Processing (Completed)

### Plan

(Pivoted architecture to meet ISRO’s strict GNSS-denied/Offline requirement) 1. Edge AI: Discard the Python backend dependency for live runs. 2. WebAssembly Integration: Embed ONNX Runtime Web (ort.min.js) into the PWA. 3. Local Navigation Engine: Write a Javascript-based Edge Engine (offline_engine.js) to process sensor buffers, run the ONNX AI model, and perform Dead Reckoning directly on the mobile device CPU. 4. Offline Caching: Update the Service Worker to cache the AI model (model.onnx) and all assets for Airplane Mode execution.

### What We Did

- Installed onnx and onnxscript and exported the TCN model to simulator/model.onnx.

- Completely refactored the web app to run autonomously without any server.

- The system now functions entirely on the edge (mobile phone), capturing data, running the neural network, and plotting the trajectory even when all networks (Wi-Fi, Cellular) are disabled.

## Phase 5: Hardened Sensor Pipeline & Edge EKF (Completed)

### Plan

- Sensor Pipeline: Build a robust preprocessor with non-linear median filtering to remove pothole/shock vibrations from IMU data.

- ML Augmentations: Introduce continuous Gaussian noise, impulsive noise, speed variation, and random heading misalignment during AI model training to guarantee generalization.

- Edge Engine Rewrite: Port the entire Python EKF (15-state), Map Matching, ZUPT, and NHC logic to purely functional JavaScript for execution inside the PWA without external backends.

### What We Did

- Developed matrix.js and ekf.js to run a fully optimized 15-state Extended Kalman Filter natively in the browser.

- Ported Geometric Map Matching to map_matcher.js for offline snapping to road networks.

- Upgraded src/models/trainer.py with rigorous batch augmentations.

- Upgraded src/data/preprocessor.py with median filtering to protect the velocity inference from violent mechanical shocks.

## Current Position & Readiness

Current State: System Ready for Field Testing

We have successfully built a full-stack, AI-powered Intelligent Dead Reckoning system that meets the extreme operational constraints of the ISRO problem statement.

### Key Capabilities Currently Active:

- Zero-Connectivity Operations: The entire application is a self-contained PWA that installs on the device and operates in Airplane mode.

- Edge Inference: The PyTorch model is running in WebAssembly on the mobile CPU.

- Full Edge EKF: A 15-state Extended Kalman Filter, complete with ZUPT, NHC, and Map Matching runs natively in JS.

- Seamless Handover: The system initializes with GNSS (if available) and seamlessly falls back to AI-driven Dead Reckoning when GNSS is lost.

### Next Steps (Optional/Future Work):

- Field Data Collection: Collect real-world IMU data on specific vehicles to fine-tune the ONNX model.

- Native Android App: If browser sensor limitations become a bottleneck, port the WebAssembly solution to a native Android APK using React Native or Flutter.