# Navigators IDR — Project Requirements Document (PRD)

## 1. Overview

The Navigators Intelligent Dead Reckoning (IDR) system provides continuous vehicle navigation using smartphone sensors during GNSS outages.

## 2. Functional Requirements

- FR1: Collect IMU data (Accel/Gyro) at minimum 50Hz (target 100Hz).

- FR2: Detect GNSS loss and seamlessly switch to dead reckoning.

- FR3: Estimate 2D vehicle velocity using a deep learning model.

- FR4: Fuse AI velocity with IMU using a 15-state EKF.

- FR5: Run completely offline without cellular or backend API dependencies in the final production app.

## 3. Non-Functional Requirements

- NFR1: End-to-end latency per window must be under 100ms (10Hz). Current ONNX benchmark: ~0.7ms.

- NFR2: System must not leak ground truth data into the estimator during validation.

- NFR3: The application must be lightweight and battery-efficient on mid-range smartphones.

## 4. Excluded Scope

- Connecting to vehicle OBD-II ports.

- Utilizing external CAN bus data or speedometer ticks.

- Cloud-based inference (strictly prohibited for the final runtime).